# REQUIRED information

- Data Description

Author: Bruno Christoff

Title: Design of Auxiliary Systems: Fuel Flow Estimation.  Dataset for NH3CRAFT Project: Parametrized Test-Benchmark with Dataset of Desktop Studies.

Abstract of dataset: This dataset is part of Deliverable D5.4: Parametrized Test-Benchmark with Dataset of Desktop Studies. It was generated using a MATLAB/Simulink model developed for a digital design platform supporting the conceptual development of ammonia-fueled ships. The dataset is used for parametric studies focused on the preliminary design of auxiliary systems. The model estimates the ammonia mass flow required by the vessel’s combustion engine based on its nominal power and engine type (2-stroke or 4-stroke). Engine power can be provided directly or estimated from the vessel’s size and type. The estimated mass flow is assumed constant throughout the auxiliary system and serves as a basis for sizing other auxiliary components.

Type of Data: Dataset

- Extended Data Description-Optional

Contributing persons: Georgios Tzortzinis

References to related material: In supplement to NH3CRAFT Deliverable: PARAMETRIZED-TEST BENCHMARK WITH DATASET OF DESKTOP STUDIES

Description of the Data: The dataset is provided in raw text file named fuelFlow. The file contains a header followed by a table of values. The header specifies the input parameters—vessel size and vessel type—as well as the output variables: the estimated engine power and the corresponding estimated ammonia mass flow required to operate the combustion engine.

Assumptions:

The estimated ammonia mass flow is based on the main engine’s fuel consumption and is assumed to remain constant across all defined auxiliary systems.

Pressure losses and other factors affecting mass flow are not considered.

The fuel supply pressure and temperature are fixed.

Engine power is estimated based on the vessel’s type and size.

Input Variables:

Size: indicative vessel size: Small, Medium, or Large.

Type: vessel type used in the analysis: RoPax, Double-Ended Ferry, Containership, Bulk Carrier, or Tanker.

Output Variables:

Power: indicative main engine power, in kilowatts (kW).

Fuel flow: estimated ammonia mass flow to the main engine, in kilograms per hour (kg/h).

Documentation of the data:

Date of data creation: 02.2025

Software:

Production: MATLAB/Simulink

Viewing: Notepad

Research Instruments: Not applicable

- Open access Publication:

Please specify the publication mode: Immediately