# REQUIRED information

- Data Description

Author: Bruno Christoff

Title: Design of Auxiliary Systems: LP pumps Dimensioning.  Dataset for NH3CRAFT Project: Parametrized Test-Benchmark with Dataset of Desktop Studies.

Abstract of dataset: This dataset is part of Deliverable D5.4: Parametrized Test-Benchmark with Dataset of Desktop Studies. It was generated using a MATLAB/Simulink model developed for a digital design platform supporting the conceptual development of ammonia-fueled ships. The dataset is used for parametric studies focused on the preliminary design of auxiliary systems. It refers to the low-pressure (LP) pumps used to transfer ammonia in liquid form from the storage tanks to the service tank, located within the fuel preparation room. The LP pump sizing is based on the required pressure difference, pump efficiency, and volume flow. The main outputs include estimations of required power, rated flow per pump, and the number of pumps needed to meet specified operational conditions.

Type of Data: Dataset

- Extended Data Description-Optional

Contributing persons: Georgios Tzortzinis

References to related material: In supplement to NH3CRAFT Deliverable: PARAMETRIZED-TEST BENCHMARK WITH DATASET OF DESKTOP STUDIES

Description of the Data: The dataset is provided in three raw text files named lpPump1, lpPump2, and lpPump3. Each file contains a header followed by a table of values. The header specifies the input parameters—vessel size, vessel type, assumed differential pressure across the pump, and pump efficiency—as well as the output variables: estimated power requirement, rated flow per pump, and the number of LP pumps required.

Assumptions:

Calculations are based on performance data from a commercial pump manufacturer.

The number of pumps indicated in the output does not account for redundancy; to ensure redundancy, twice the calculated number of pumps should be installed.

A pump efficiency is assumed to simplify the estimation process and provide a preliminary sizing. For detailed design, consultation with manufacturers is recommended.

Input Variables:

Size: indicative vessel size: Small, Medium, or Large.

Type: vessel type used in the analysis: RoPax, Double-Ended Ferry, Containership, Bulk Carrier, or Tanker.

Pressure: assumed pressure difference across the LP pump, in bar.

Efficiency: assumed pump efficiency.

Output Variables:

Power: required power of the pump to meet the specified operational parameters, in horsepower (HP).

Rated Flow: rated flow of the pump at optimal operating conditions, in gallons per minute (GPM).

Number of Pumps: number of pumps required to meet the operational parameters (excluding redundancy).

Documentation of the data:

Date of data creation: 02.2025

Software:

Production: MATLAB/Simulink

Viewing: Notepad

Research Instruments: Not applicable

- Open access Publication:

Please specify the publication mode: Immediately