# REQUIRED information

- Data Description

Author: Bruno Christoff

Title: Design of Auxiliary Systems: Buffer Tank Dimensioning.  Dataset for NH3CRAFT Project: Parametrized Test-Benchmark with Dataset of Desktop Studies.

Abstract of dataset: This dataset is part of Deliverable D5.4: Parametrized Test-Benchmark with Dataset of Desktop Studies. It was generated using a MATLAB/Simulink model developed for a digital design platform supporting the conceptual development of ammonia-fueled ships. The dataset is used for parametric studies focused on the preliminary design of auxiliary systems. It presents data for dimensioning the buffer tank, specifically calculating the required volume to operate the vessel for a specified number of hours at a given Nominal Continuous Rating (NCR).

Type of Data: Dataset

- Extended Data Description-Optional

Contributing persons: Georgios Tzortzinis

References to related material: In supplement to NH3CRAFT Deliverable: PARAMETRIZED-TEST BENCHMARK WITH DATASET OF DESKTOP STUDIES

Description of the Data: The dataset is provided in three raw text files named bufferTank1, bufferTank2, and bufferTank3. Each file contains a header followed by a table of values. The header specifies the input parameters—vessel size, vessel type, the assumed number of hours for continuous operation, and the Nominal Continuous Rating (NCR)—as well as the output variable: the required volume of the buffer tank.

Assumptions:

The total volume of the buffer tank is assumed to be sufficient for the vessel to operate at its Nominal Continuous Rating (NCR) for the specified number of hours.

The ammonia density is calculated based on the operating temperature and pressure of the vessel’s main engine (M/E).

Input Variables:

Size: indicative vessel size: Small, Medium, or Large.

Type: vessel type used in the analysis: RoPax, Double-Ended Ferry, Containership, Bulk Carrier, or Tanker.

Hours: number of hours of operation for which the buffer tank is designed to hold.

NCR: percentage used to calculate the Nominal Continuous Rating (NCR) as a fraction of the vessel's Maximum Continuous Rate (MCR).

Output Variables:

Volume: required volume of the buffer tank to operate the vessel for N hours at a user-specified Nominal Continuous Rating (NCR), in cubic meters (m³).

Documentation of the data:

Date of data creation: 02.2025

Software:

Production: MATLAB/Simulink

Viewing: Notepad

Research Instruments: Not applicable

- Open access Publication:

Please specify the publication mode: Immediately