# REQUIRED information

- Data Description

Author: Bruno Christoff

Title: Design of Auxiliary Systems: HP Pumps Dimensioning Dimensioning.  Dataset for NH3CRAFT Project: Parametrized Test-Benchmark with Dataset of Desktop Studies.

Abstract of dataset: This dataset is part of Deliverable D5.4: Parametrized Test-Benchmark with Dataset of Desktop Studies. It was generated using a MATLAB/Simulink model developed for a digital design platform supporting the conceptual development of ammonia-fueled ships. The dataset is used for parametric studies focused on the preliminary design of auxiliary systems. it presents the data for dimensioning the high-pressure (HP) pumps, with input parameters including the assumed pressure differential and volume flow. The outputs include the required pump power, rated flow at optimal operating conditions, and the number of pumps needed to meet the operational parameters, ensuring the pump system supports the main engine (M/E) operational profile.

Type of Data: Dataset

- Extended Data Description-Optional

Contributing persons: Georgios Tzortzinis

References to related material: In supplement to NH3CRAFT Deliverable: PARAMETRIZED-TEST BENCHMARK WITH DATASET OF DESKTOP STUDIES

Description of the Data: The dataset is provided in three raw text files named hpPump1, hpPump2, and hpPump3. Each file contains a header followed by a table of values. The header specifies the input parameters—vessel size and vessel type, and the assumed pressure differential across the high-pressure pump—as well as the output variables: required pump power, rated flow at optimal operating conditions, and the number of HP pumps needed to meet the specified operational parameters.

Assumptions:

Calculations are based on performance curves provided in manufacturer catalogues.

If the input flow rate exceeds the manufacturer’s recommended maximum output flow, it is assumed that the input flow is divided into two equal parts, requiring two identical pumps.

Redundancy is not considered in the calculation of the number of pumps.

Input Variables:

Size: indicative vessel size: Small, Medium, or Large.

Type: vessel type used in the analysis: RoPax, Double-Ended Ferry, Containership, Bulk Carrier, or Tanker.

Pressure: assumed pressure differential across the high-pressure pump, in bar.

Output Variables:

Power: required power of the pump to meet the specified operational parameters, in kilowatts (kW).

Flow: rated flow of the pump at optimal operating conditions, in cubic metres per hour (m³/h).

Number Pumps: number of pumps required to meet the operational parameters (excluding redundancy).

Documentation of the data:

Date of data creation: 02.2025

Software:

Production: MATLAB/Simulink

Viewing: Notepad

Research Instruments: Not applicable

- Open access Publication:

Please specify the publication mode: Immediately