# REQUIRED information

- Data Description

Author: Bruno Christoff

Title: Design of Auxiliary Systems: Heater Dimensioning.  Dataset for NH3CRAFT Project: Parametrized Test-Benchmark with Dataset of Desktop Studies.

Abstract of dataset: This dataset is part of Deliverable D5.4: Parametrized Test-Benchmark with Dataset of Desktop Studies. It was generated using a MATLAB/Simulink model developed for a digital design platform supporting the conceptual development of ammonia-fueled ships. The dataset is used for parametric studies focused on the preliminary design of auxiliary systems. It presents the data for dimensioning the heaters, where heat transfer occurs between ammonia and a water-glycol mixture. The heater is responsible for raising the ammonia temperature to meet the operational requirements defined by the main engine. The calculations consider the variation in the inlet and outlet temperatures of the water-glycol mixture, the ammonia flow rate through the heater, and its operating conditions. The outputs include the energy required to heat the ammonia and the corresponding flow rate of the water-glycol mixture needed to ensure the necessary heat transfer.

Type of Data: Dataset

- Extended Data Description-Optional

Contributing persons: Georgios Tzortzinis

References to related material: In supplement to NH3CRAFT Deliverable: PARAMETRIZED-TEST BENCHMARK WITH DATASET OF DESKTOP STUDIES

Description of the Data: The dataset is provided in three raw text files named heater1, heater2, and heater3. Each file contains a header followed by a table of values. The header specifies the input parameters—vessel size and type, assumed temperature difference of the water-glycol mixture, and its specific heat capacity (cp)—as well as the output variables: the energy required to heat the ammonia and the required flow rate of the water-glycol mixture to achieve the necessary heat transfer.

Assumptions:

The thermodynamic states of ammonia entering and leaving the heater are assumed constant.

No pressure loss is considered in the system.

The total heat is calculated as an estimate based on an idealised energy balance.

Heat loss to the surroundings is neglected in the calculations.

Input Variables:

Size: indicative vessel size: Small, Medium, or Large.

Type: vessel type used in the analysis: RoPax, Double-Ended Ferry, Containership, Bulk Carrier, or Tanker.

Temp: Assumed temperature difference for the water-glycol mixture, in degrees Celsius (°C).

cp: Assumed specific heat capacity of the water-glycol mixture, in Joules per kilogram per degree Celsius (J/kg·°C).

Output Variables:

Power: The energy required to heat the ammonia to meet the main engine (M/E) manufacturer's specifications, in kilowatts (kW).

Flow: The required flow rate of the water-glycol mixture to achieve the desired heat transfer, in kilograms per second (kg/s).

Documentation of the data:

Date of data creation: 02.2025

Software:

Production: MATLAB/Simulink

Viewing: Notepad

Research Instruments: Not applicable

- Open access Publication:

Please specify the publication mode: Immediately