# REQUIRED information

- Data Description

Author: Bruno Christoff

Title: Design of Auxiliary Systems: Recirculation Tank Dimensioning.  Dataset for NH3CRAFT Project: Parametrized Test-Benchmark with Dataset of Desktop Studies.

Abstract of dataset: This dataset is part of Deliverable D5.4: Parametrized Test-Benchmark with Dataset of Desktop Studies. It was generated using a MATLAB/Simulink model developed for a digital design platform supporting the conceptual development of ammonia-fueled ships. The dataset is used for parametric studies focused on the preliminary design of auxiliary systems. This dataset presents the data for dimensioning the recirculation tank, which separates liquid ammonia from nitrogen during the start-up and shutdown phases of the main engine, and collects unburned ammonia vapor. The calculations are based on the assumed percentage of recirculated fuel, the Nominal Continuous Rating (NCR), and a multiplier factor that acts as a safety margin. The resulting output is the estimated volume of the recirculation tank.

Type of Data: Dataset

- Extended Data Description-Optional

Contributing persons: Georgios Tzortzinis

References to related material: In supplement to NH3CRAFT Deliverable: PARAMETRIZED-TEST BENCHMARK WITH DATASET OF DESKTOP STUDIES

Description of the Data: The dataset is provided in three raw text files named recircTank1, recircTank2, and recircTank3. Each file contains a header followed by a table of values. The header specifies the input parameters—vessel size, vessel type, assumed percentage of recirculated ammonia, percentage used for calculating the Nominal Continuous Rating (NCR), and the safety factor applied to the recirculated fuel volume. The output variable is the required volume of the recirculation tank.

Assumptions:

The total volume of the recirculation tank is assumed to be sufficient to support the vessel’s operation at its Nominal Continuous Rating (NCR), based on the specified percentage of recirculated fuel.

A safety factor is applied to the calculated recirculated fuel volume to ensure additional capacity, accounting for operational uncertainties or design margins.

Input Variables:

Size: indicative vessel size: Small, Medium, or Large.

Type: vessel type used in the analysis: RoPax, Double-Ended Ferry, Containership, Bulk Carrier, or Tanker.

Percentage: Percentage of fuel assumed to be recirculated during M/E operation.

NCR: Percentage used to calculate the Nominal Continuous Rating (NCR) in relation to the vessel’s nominal power.

Number: Safety factor applied to the calculated volume of recirculated fuel.

Output Variables:

Volume: required volume of the recirculation tank, in cubic meters (m3).

Documentation of the data:

Date of data creation: 02.2025

Software:

Production: MATLAB/Simulink

Viewing: Notepad

Research Instruments: Not applicable

- Open access Publication:

Please specify the publication mode: Immediately