# REQUIRED information

- Data Description

Author: Bruno Christoff

Title: Design of Auxiliary Systems: Nitrogen Supply System Dimensioning.  Dataset for NH3CRAFT Project: Parametrized Test-Benchmark with Dataset of Desktop Studies.

Abstract of dataset: This dataset is part of Deliverable D5.4: Parametrized Test-Benchmark with Dataset of Desktop Studies. It was generated using a MATLAB/Simulink model developed for a digital design platform supporting the conceptual development of ammonia-fueled ships. The dataset is used for parametric studies focused on the preliminary design of auxiliary systems. This dataset presents the data for dimensioning the nitrogen supply system, which is primarily used for inerting storage tanks and piping before bunkering or maintenance, and for purging liquid lines after bunkering operations or when the main engine is switched off. The calculations are based on the dimensions of supply and return lines, engine volume, the number of annual trips, purging frequency per trip, purging system pressure, and nitrogen buffer tank pressure. The dimensioning yields the number of nitrogen bottles required, the total nitrogen capacity needed, and the estimated annual operating time of the nitrogen generator.

Type of Data: Dataset

- Extended Data Description-Optional

Contributing persons: Georgios Tzortzinis

References to related material: In supplement to NH3CRAFT Deliverable: PARAMETRIZED-TEST BENCHMARK WITH DATASET OF DESKTOP STUDIES

Description of the Data: The dataset is provided in three raw text files named nitrogen1, nitrogen2, and nitrogen3. Each file contains a header followed by a table of values. The header specifies the input parameters—vessel size, vessel type, and the number of purging times per trip—along with the output variables: the number of nitrogen bottles required, the purging capacity per trip, the nitrogen capacity required, the nitrogen purge per year, and the selected nitrogen purge volume considering a safety factor.

Assumptions:

A safety factor (SF) of 1.5 is applied to the estimated nitrogen purge operation per year to account for additional safety margins.

Input Variables:

Size: indicative vessel size: Small, Medium, or Large.

Type: vessel type used in the analysis: RoPax, Double-Ended Ferry, Containership, Bulk Carrier, or Tanker.

Purg Trip: The number purging operations per trip.

Output Variables:

#Bottles: The number of nitrogen bottles required for the purging operations.

Purging_cap: Number of purging operations per trip.

N2_cap:   The total nitrogen capacity needed for purging operations, in cubic meters (m3).

N2_Purg_Year:  Running time of the N2 generator, in hours per year (h/year).

Safety: Running time considering a safety factor, in hours per year (h/year).

Documentation of the data:

Date of data creation: 02.2025

Software:

Production: MATLAB/Simulink

Viewing: Notepad

Research Instruments: Not applicable

- Open access Publication:

Please specify the publication mode: Immediately